| Safe Work Method Statement Mobile Crane – General Activities 35T Liebherr 55T Grove | Completion Date: 05/06/2014 Review Dates: 30/03/2015 30/03/2016 | Approved Craig Bottom: (signature): Date: Nominated Supervisor: Craig Bottom | Last Reviewed: 30/03/2016 |
| --- | --- | --- | --- |
| Risk Assessment conducted by: Craig Bottom This Work Method Statement has been developed in consultation with our employees and where required amended then endorsed by those employees involved with these activities. This WMS should be read in conjunction with the induction and site instructions | Future Review Date: March 2018 | Client Name: Project Name: Project Address: |  |
|  | Refer to SMP No: |  |  |

Hierarchy of Control Assessment Matrix

To assist in demonstrating the application 	of the Hierarchy of Controls associated with risks and hazards identified in this Work Method Statement, the elements of Controls have been numbered for ease of reference throughout the document.

| (1) ELIMINATION | Remove the hazard from the workplace |
| --- | --- |
| (2) SUBSTITUTION | Use something less hazardous |
| (3) ISOLATION | Use barriers to shield or isolate the hazard from the person |
| (4) ENGINEERING | Design and install equipment to counteract the hazard |
| (5) ADMINISTATIVE | Implement procedures or permits to minimise risk exposure |
| (6) PERSONAL PROTECTIVE EQUIPMENT (PPE) | Have people trained to wear and use protective equipment |

HIGH RISK CONSTRUCTION WORK

| Plant Equipment | Tower Crane Mobile Crane Franna | Forklift Alimak Hoist Elevating Work Platform | Scissor Lift Platform Platform Ladder Vehicle Loading Crane |  |
| --- | --- | --- | --- | --- |
| Licences Permits | Tower Crane Mobile Crane Franna Dogging | Forklift Alimak Hoist Elevating Work Platform Rigging | Scissor Lift Platform Vehicle Loading Crane Harness Permit Scaffolding | Work At Height Electrical Connection Hot Work |
| Training Instruction | Construction Induction Site Induction Specific Plant Training | Manual Tasks PPE Work At Heights | Confined Spaces TTM |  |
| Maintenance Checks | Tower Crane Mobile Crane Franna | Forklift Alimak Hoist Elevating Work Platform | Scissor Lift Platform Platform Ladder Vehicle Loading Crane | Lifting/Rigging Gear Electrical Testing/Tagging Safety Harness |
| PPE | Safety Boots Hi-Vis Clothing Safety Helmut Respiratory Work Safety Glasses Safety Harness Protection Gloves &/or Face Shield |  |  |  |
| MSDS Required | If YES, list hazardous substances & append MSDS |  |  |  |
| Other Requirements |  |  |  |  |

| Item | Processes/ Task | Hazard (include in the description whether the condition is Normal, Abnormal or Emergency) | Risk Ranking |  |  | Potential Controls | Residual Risk |  |  | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | L | R |  | C | L | R |  |
| 1 | Communications and Consultation | Hazard Being unfamiliar with Cooma Crane Hire specific site requirements and hazards. Risk Incurring an injury by being exposed to unknown site hazards |  |  |  | On arrival at site for the first time you must contact the client supervisor. Your client supervisor will make arrangements for your site specific induction and identify key personnel relevant to your site activities which may include: - Site Manager - Site OHS Officer/HSR - Site Supervisor - First Aid person - Emergency Evacuation 5 Administrative Control |  |  |  | Crane Operator Rigger & Dogman |
| 2 | Site registration and induction | Prior to commencing work the first time you attend a site, you must attend a Cooma Crane Hire site induction and at any time the client conducts another induction. You must hold the appropriate High Risk Work Licence and hold a current Construction Industry Induction card. |  |  |  | Re-assess risks and site conditions to ensure there are no changes that may affect the safe operation of the crane. |  |  |  | Crane Operator Rigger & Dogman |
| 3 | Toolbox Talks |  |  |  |  | You must attend a toolbox talk prior to carrying out the work and record your attendance in the Pre-Job Inspection and Toolbox Talk section of the Cooma Crane Hire daily Tax Invoice. |  |  |  | Crane Operator Rigger & Dogman |
| 4 4 | Personal Protective Equipment (PPE) Cont. | Hazard Being struck/crushed by powered mobile plant Risk Potential for personal injury from mobile plant when unseen by operator or not being aware of moving plant. Hazard Excessive noise levels, impact on feet and foreign bodies in eyes Risk Personal injury from the effects of noise, foot injury, hand or eye injury. | 3 | 3 | H | PPE as determined by the principal contractor must be worn at all times in accordance with site instructions. Safety helmets must be worn when working with cranes. PPE may be safety hat, boots, glasses, hi-vis clothing, gloves. Refer AS1067 Safety Glasses, AS1800 Safety Helmets, AS1891.4 Industrial Fall Arrest Devices, AS2161.1 Protective Gloves, AS2210.1 Protective Footwear, AS1270 Hearing Protectors 6 Personal Protective Equipment | 2 | 1 | L | Crane Operator Rigger & Dogman |
| 5 | REVERSING A MOBILE CRANE IN TIGHT WORK LOCATIONS TO SET UP IN PREPARATION FOR WORK | Hazard Dogger/Rigger guiding mobile crane from behind becoming caught between crane and a stationary object. Risk Personal injury or death from crushing injuries. | 5 | 3 | E | 1.0 Check radio communication before moving crane. 2.0 Discuss a backup means of communication such as visual or using a whistle. Check that the whistle can be heard by operator. 3.0 Use additional spotters placed in locations where the operator can see them. 4.0 Person at rear of crane must where room permits maintain a distance of 5M from the rear of the moving crane. 5.0 When reversing the crane up to a load, wall or other stationary object the person in charge must stop the crane 3m from the object then place themselves at a corner of the crane in view of the operator and direct the crane backwards to the set-up position using hand signals. Refer Code of Practice – Moving Plant on construction sites 2004 with particular reference to 3.1 Common hazards involving powered mobile plant. 5 Administrative Control | 3 | 3 | H | Rigger & Dogman |
| 6 | Tools and equipment | Hazard Damaged or poorly maintained tools and equipment. Risk May lead to injury or damage to plant | 4 | 3 | E | Ensure a visual inspection is carried out on all tools and equipment prior to use. All slings, chains, shackles and other specialist lifting equipment must be in working condition, undergo regular inspection by an authorised inspector and have appropriate WLL tags attached 5 Administrative Control | 3 | 2 | M | Crane Operator Rigger & Dogman |
| 7 | Manual Tasks | Hazard Heavy lifting equipment required to be manhandled Risk Possible serious long term injuries to back or other parts of body. Fatigue, stress or violence | 3 4 | 3 3 | H E | Assess weights of loads to be lifted or dragged. All employees received Manual tasks training including team lifting techniques during the Cooma Crane Hire induction. 5 Administrative Control Supervisors and employees to monitor each other for signs of fatigue or stress and where necessary have the effected person removed from the workplace. 5 Administrative Control | 2 1 | 3 1 | M L | Crane Operator Rigger & Dogman |
| 8 8 | Inspection of work area Cont. Inspection of work area – Ground Stability considerations | Hazard Hidden services such as electricity, gas, water and communication lines Risk Collapse of ground under carrier or crane stabilisers when crane in use causing damage to underground services Hazard Contact with overhead electrical services. Risk Working too close to electrical services Hazard Working too close to battered banks, constructed retaining walls or on suspended concrete slabs. Risk Potential for a collapse of ground or structure resulting in crane roll over. Hazard Unauthorised persons or other workers straying into danger area of slewing crane. Risk Crushing or impact injuries to workers and general public from unexpected movement of powered mobile plant. Hazard Road traffic passing in close proximity of work location. Risk Collision between traffic and mobile crane. Hazard Penetrations – Stepping or falling into holes or unprotected openings in ground or work decks. Risk Risk of injury by tripping on an open penetration or falling through | 4 4 4 4 4 3 4 | 3 3 3 3 3 3 3 | E E E E E H E | General Crane Work: At the beginning of the job enquire about hidden services with client. If necessary ring Dial Before You Dig to obtain information or contact ACTPLA. Contractor Sites: Inspect area prior to setting crane up to identify hidden hazards such as underground services. Refer: AS2550.1-2011 Cranes-General Requirements (Sec 4.2) Crane Standing Contact must be made with a site representative to enquire about the presence of underground services. Look for overhead services and discuss with site contact or client to ensure visible protection is suitable for working in close proximity to electrical services. 5 Administrative Control Obtain (where possible documented) engineering advice on structural capabilities when working near retaining walls, excavations or on concrete decks. Do not establish crane close than a 450 angle from the base of a sloping embankment. [ Set up 2m back from a 1m deep hole or further if ground looks suspect] Check operating area to ensure adequate clearances for swing arc of crane. Rigger/Dogger to carry out regular checks to ensure the clearance is maintained. Ensure that an effective Temporary Traffic Control (TTM) is in place before commencing work. Riggers/Dogmen to escort vehicles when reversing and not place themselves at risk of being crushed. 3 Isolation Where necessary make sure that appropriate measures are in place for traffic control. Persons setting up or engaged in traffic control duties must be trained in such activities. Make sure that crane is equipped with an effective spill kit to manage oil or fuel spills. Inspect work area for holes and penetrations and do not work near these hazards. Stop working in area if hazards cannot be managed. Do not lift or remove penetration covers. 4 Engineering After wet conditions or on hardstands not capped, supervisor to assess ground conditions prior to any work. | 2 2 2 2 2 2 1 | 2 2 2 2 2 2 2 | L L L L L L L | Crane Operator Rigger & Dogman. Site contact Client Crane Operator Rigger & Dogman. Site contact Crane Operator Rigger & Dogman. |
| 9 | Inspection of all lifting equipment | Hazard Defective or worn equipment can result in failure. Risk Loss of load or part load | 4 | 3 | E | Make sure that all lifting equipment has been inspected annually by a certified inspection contractor who provides a documented report. Refer: AS3775.1 Chain Slings Lifting equipment must be inspected every day prior to packing away. A changed crane crew must inspect equipment prior to leaving the yard to ensure it is all accounted for and is in good working order. Identified damaged or defective equipment should be removed from service and have an “Out Of Service” tag attached to flag that it must not be used. Use engineered lifting points. All bolted connections to be snug tight. | 2 | 3 | M | Dogman Rigger |
| 10 | Check the Working Load Limit (WLL) as described on a crane load chart, specific to each individual crane. | Hazard Crane being overloaded Risk Crane operating outside its safety limits. Cranes being overloaded may cause structural damage resulting in failure and/or overturning the crane. There is also a risk to the safety of persons and potential damage to the load or other plant and equipment. | 4 | 3 | E | Riggers/Dogmen to determine weight of the load being lifted. Check where the load is being lifted from and to and again refer to the load chart to make sure the lift is within the crane WLL. Riggers/Dogmen to select the appropriate lifting equipment. Riggers/Dogmen must refer to the “Cranes – Pre-Cast Concrete Placement” work method statement when lifting pre-cast concrete products or when a dual crane lift is being contemplated. 5 Administrative | 2 | 2 | L | Authorised inspection contractor Crane Operator Rigger & Dogman. |
| 11 | Communication between crane operator and Rigger/Dogger | Hazard Loss of communication during a lift. Risk A potential for incorrect movement of the crane resulting in damage to load or crush injury. | 4 | 3 | E | Crane crew to hold a discussion prior to lifting the load to make sure all information has been shared and understood including emergency procedure for loss of radio during a lift. Rigger/Dogger make sure clear line of sight to operator and give clear signals. When working out of sight of operator Riggers/Dogmen must carry a whistle and make sure there is a spare charged battery available for the radios. Refer: AS2550.1 2011 Cranes-General Requirements (Sec 6.13) Communication 5 Administrative | 3 | 2 | M | Crane Operator Rigger & Dogman |
| 12 | Unfastening loads in preparation for lift | Hazard Loose parts of load becoming destabilised and falling Risk A potential for workers or others close by being struck and injured | 3 | 3 | H | Rigger/Dogman to assess load and discuss with truck driver before undoing restraints. Rigger/Dogman to stand well clear of part of load being lifted. If there is a risk of load moving under foot, Rigger/Dogman to get off truck and stand on the ground. For loads where it is difficult gain access to attach slings, Rigger/Dogman must use suitable means of access. This may include (but not limited to) using platform ladders or an elevating work platform. 5 Administrative | 3 | 2 | M | Riggers Dogman |
| 13 | Slinging Loads – General | Hazard Lifting or dragging heavy lifting equipment. Risk Hand injuries or risk of muscular injury when attempting to lift heavy weights Hazard Being hit by free swinging chains or slings. Risk A potential for workers or others close by being struck and injured Hazard Being hit by free swinging chains or loss of load. Risk A potential for workers or others close by being struck and injured | 3 2 4 | 3 4 3 | H H E | All loads to have tag lines. Use crane or other lifting equipment to lift heavy chains. Use lifting techniques as discussed in Manual Tasks training. Refer: Work Health and Safety(Hazardous Manual Tasks) Code of Practice 2011 Keep hands clear of nip points and from in between parts of loads. Use appropriate gloves. Hang excess chains on main ring or use safety latch to attach to lifting chains Stand clear and keep hands clear of load when taking up slack in chains or slings. Cardinal Rule: Never be under suspended load When lifting containers provided with four lift points ALL four chains must be attached to keep load stable. Do not lift bins where a part of the load is protruding over the edge of the bin. Do not lift bulk bags by the handles provided. Place in a bin or on a pallet and use a pallet cage. | 2 2 2 | 3 2 2 | M L L | Riggers Dogman Crane Operator Riggers Dogman Crane Operator |
| 14 | Lifting loads from truck | Hazard An uncontrolled load swing Risk Being struck or crushed by load Hazard Failure of lifting gear from not being compatible Risk Risk of a load or pieces of mixed or odd loads falling. Uneven or unbalanced loads slipping in chains or slings and dropping Risk of riggers/doggers falling when slinging high or unusual loads | 3 4 3 4 4 | 3 3 3 3 3 | H E H E E | Cardinal Rule: Never be under suspended load Rigger/Dogger to make sure crane hook is directly over load or balance point. Select and use appropriate matching lifting equipment that is in good condition. Refer to Franna Pre-cast Panels WMS when lifting pre-cast concrete products and make sure that appropriate specialist equipment is used. Rigger/Dogman to assess the need to apply additional binding restraints Rigger/Dogman to lift load clear of ground and stop crane to inspect for possible loose objects beneath the load Rigger/Dogman to lift load clear and stop crane to assess load balance. Lower load and reposition chains or slings to balance as necessary Rigger/Dogman must complete an assessment on the best way of gaining safe access to attach or remove lifting equipment and document in a JSA. | 1 1 1 1 2 | 3 3 2 2 2 | L L L L L | Riggers Dogman Riggers Dogman Riggers Dogman Riggers Dogman Crane operator |
| 15 | CHANGES TO WORK PRACTICES OR CHANGES TO LIFTING PROCEDURES | Failure to understand the capability of the crane or lifting equipment. Loss of communication during a lift with the potential for incorrect movement of the crane resulting in damage to load or crush injury. | 4 | 3 | E | Cardinal Rule: Never be under suspended load Whenever there is an unplanned change to an agreed method of how the work is to be completed the crane crew must discuss the changes and agree to a new method. Such discussion and agreed changes must be documented on a Toolbox Talk form and signed by all participants. For complex or difficult changes contact the plant scheduler for advice or assistance. | 2 | 3 | M | Riggers Dogman Crane operator |
| 16 | Environmental Fuel or oil spills Note: Mobile cranes operate using a diesel engine that drives a hydraulic oil pump. Note: Copies of Material Safety Data Sheets that are specific to diesel and hydraulic oil must be kept in mobile crane carrier for reference | Hazard Hydraulic oil or fuel leaks when travelling to/from work locations or on site. Risk Environmental damage | 3 | 3 | H | Crane crew to stop work and assess the need to contain leaks and use spill kit when travelling on roads. Place environment socks in gutters where there is a potential to pollute drainage system In the event of a leak when on site, stop and turn crane off. Follow any site specific instructions for environmental incidents Notify client immediately for assistance and notify B & D Crane Hire management. Handle and dispose of waste in accordance with MSDS 3 Isolation/5 Administrative | 2 | 2 | L | Riggers Dogman Crane operator & Client |

|  | Processes/Tasks | Hazard | Risk ranking |  |  | Potential Controls | Residual Risk |  |  | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

This Work Method Statement has been developed through consultation with our employees and has been read and signed by all employees involved with any of the activities mentioned above.

We the undersigned acknowledge that we were involved in the development of this Work Method Statement and are consulted regularly when there is requirement for an addition of identified hazards for the operation of mobile cranes. We also acknowledge that we have been trained in the tasks identified above. Employee qualifications and job roles are defined below.

| Name | Signature | Qualification/Ticket Number | Job Role |
| --- | --- | --- | --- |
| Craig Bottom |  |  | Supervisor/Crane Operator |
| Tom Wassink |  |  | Senior Crane Operator |
| Stefan Berner |  |  | Dogman |
| Ben Hayden |  |  | Dogman |

| Personnel qualifications and experience required to complete the task: | Specific training required to complete this task: | Engineering Details/Certificates/ Approvals Where applicable: |
| --- | --- | --- |
| Crane operator certificate of competency | High Risk Work Licence |  |
| Rigger certificate of competency | High Risk Work Licence |  |
| Dogger certificate of competency | High Risk Work Licence |  |
| Work at heights | Work at Heights training |  |
| Manual Tasks | # Work Health and Safety (Hazardous Manual Tasks) Code of Practice 2011 |  |
|  |  | Engineering confirmation to set up crane on suspended concrete slabs |
|  |  | Design verification for special purpose lifting equipment |

PLANT, EQUIPMENT AND RELEVANT INSPECTIONS AS PER THE COOMA CRANE HIRE SAFETY MANAGEMENT PLAN

Plant and equipment include the following that may be used on site

| Crane = 250 hour service and inspection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Yearly inspection as per AS2550.1- 2002 General Requirements Sec 7.3.4.1 Refer to Section 6 of the Cooma Crane Hire Safety Management Plan |  |  |  |  |
| Yearly inspection of lifting equipment by a qualified independent contractor |  |  |  |  |

STANDARDS APPLYING TO THE WORK

AS 1319 – 1994 Safety signs for the occupational environment

AS 1353.1-1997 Flat synthetic–webbing slings – Product specification

AS 1418.1 – 2002 Cranes (including hoists and winches

AS 1666.1 – 2009 Wire rope slings – Product specification

AS 1742.3 – 2009 Traffic control devices for works on roads

AS/NZS 1891.4 – 2009 Industrial fall arrest systems and devices – Selection, use and maintenance

AS/NZS 2161.1:2000 Occupational protective gloves – Selection, use and maintenance

AS 2550.1- 2011 Cranes (safe use and general requirements)

AS 2741 – 2002 Shackles

AS 3000, Wiring Rules.

AS 3775.1/2 – 2004 Chain Slings

AS 4497.1 – 1997 Round slings – Synthetic fibre – Product specification

AS 4497.2 – 1997 Round slings – Synthetic fibre – Care and use

Legislation, Codes of Practice & Industry Guidance

Work Health and Safety Act 2011

Work Health and Safety Regulation 2011

Work Health and Safety (Hazardous Manual Tasks) Code of Practice 2011

National Standard for Plant 1010

ACT Building and Construction Industry Safety Handbook

ACT Powered Mobile Plant – Managing Risk – Control System Examples – Warning Devices

# Work Health and Safety (Hazardous Manual Tasks) Code of Practice 2011

The implementation of the HIERACHY OF CONTROLS will be as follows: Elimination; Substitution; Engineering; Administration and PPE.

In the event of an incident or a departure in the steps of this WMS, work shall cease immediately. Work will only recommence when corrective action has been implemented.

A random safety check must be carried out by a management representative to ensure that works carried out reflect the steps outlined in the above WMS.